# An Introduction to Heat Pumps

Prepared for Heat Pump DST grant RAs

## Heat Pumps: The Basics

Heat pumps can be distinguished from conventional HVAC systems in two particular ways:

- Heat pumps are a two-in-one system, and can run in both heating and cooling mode, replacing the need for an air conditioner and a furnace.

- Heat pumps are more efficient than traditional systems because they simply move heat rather than creating heat through fossil fuel combustion or electric resistance.

Generally, residential and small commercial buildings will be considering “air source heat pumps”. Geothermal heat pumps are more efficient, but have limited application due to more complex, expensive, and space-consuming installation. Air source heat pumps, or ASHP, can run completely on electricity, or have a gas backup – a system referred to as “dual fuel”. ASHP are described below.

## How They Work

Put simply, heat pumps use electricity to move heat from one place to another.  Like a kitchen refrigerator, a heat pump utilizes a refrigerant for both its cooling and heating processes.

Heat pumps are comprised of two units: an outdoor unit and an indoor unit. If a building has air ducts, the indoor unit can be connected directly to the ducts; if a building lacks air ducts, a ductless indoor unit can be utilized.

The outdoor unit and the indoor unit are connected by tubes, which are filled with the refrigerant. Modern heat pumps are able to be used even in subzero temperatures, because the refrigerant is colder than the air. While they are less efficient during the winter, they are not impacted by wind chill, and can be expected to work in temperatures as low as –10 Fahrenheit. Certain models of heat pump are specifically designated as “Cold-Climate”.

When cooling, a standard heat pump uses the refrigerant to absorb heat from indoor air, which is then passed outside when the heat is compressed and passed through an outdoor heat exchanger.

When heating, the refrigerant absorbs heat from the outdoors and vaporizes inside the sealed system.

## Benefits

Heat pumps are more energy efficient than traditional heating and cooling systems.

Because heat pumps are moving heat from one place to another, heat pumps can be anywhere between 100 – 400% efficient compared to much less efficient wood or natural gas options for heating. This efficiency tends to save the user money in the long-term.

Whereas traditional heating sources produce pollutants due to combustion, heat pumps’ use of electricity to move heat from one place to another is ultimately producing less carbon output. Of course, this will be determined by the source of that electricity wherever the heat pump is being used. For example, if a heat pump is drawing its source from a renewable energy source like solar panels or wind turbines, the net carbon output is much reduced compared to a heat pump drawing its source from a coal plant.

## Understanding Heat Pump Costs

The biggest roadblock towards pursuing a heat pump for most is the initial upfront cost. While heat pumps ultimately result in long-term savings, initial purchase and installation costs can be pricey.

Rewiring America provides the following cost estimates for “the 20th to 80th percentile of whole-home air source heat pumps”:

| SIZE | COST RANGE | MEDIAN |
| --- | --- | --- |
| 1,500 to 2,500 square foot home | $22,500 - $28,000 | $19,500 |
| 2,500 to 5,500 square foot home | $22,500 - $28,000 | $25,000 |
| 5,500+ square foot home | $26,000 - $30,000 | $29,000 |

In addition, before installing a heat pump, one should take preliminary steps to ensure maximum efficiency of the product. These steps include hiring an energy auditor, who will be able to provide detailed information about the building’s thermal envelope and ability to retain heat. If needed, hiring a weatherization contractor (insulation, sealing, etc.) may be necessary prior to installation.

It is important to note that, depending on one’s state and utility provider, they may have access to various rebate programs to assist with the purchase, installation, or maintenance of a heat pump. See AES Indiana as an example.

## Myths and Misperceptions

Heat pump and heat pump technology can be rife with myths and misperceptions. Many of these misunderstandings of heat pumps were true of older heat pump technology, but modern heat pumps have advanced beyond them.

Carbon Brief has compiled a comprehensive list of these misperceptions, which can be found here, as well as being outlined below. A full analysis of each misperception can be found on the Carbon Brief page.

Myths:

- Heat pumps don’t work in old buildings

- Heat pumps only work in highly insulated buildings

- Heat pumps only work with underfloor heating

- Heat pumps won’t work in flats (apartments)

- Heat pumps don’t work when it’s cold

- Heat pumps will always need a backup heating system to keep you warm

- Heat pumps won’t keep you warm

- Heat pumps are noisy

- Turning gas to electricity to heat via a heat pump is less efficient than burning gas in a boiler

- Heat pumps will never offset the carbon emissions resulting from making them

- Heat pumps devalue properties

- Heat pumps don’t last long

- Heat pumps are new and untested technology

Misleading:

- You will freeze during a power cut and be better off with a gas boiler

- Heat pumps cost more to run and will increase heating bills

- Heat pumps are unaffordable

- The grid cannot cope with heat pumps

- Heat pumps don’t work with microbore piping

## FAQs

The Michigan Heat Pump Collaborative provides a comprehensive FAQ section on their website, found here. Key questions and answers are found below:

How will a heat pump affect annual heating costs?

This will be different for every home. A home’s savings will depend on the current source of heating fuel, age of home, amount of insulation, among other factors. The Northeast Energy Efficiency Partnerships found that when heat pumps designed specifically for colder regions were installed in the Northeast and Mid-Atlantic regions, the annual savings are around 3,000 kWh (or $459) when compared to electric resistance heaters, and 6,200 kWh (or $948) when compared to oil systems. When displacing oil (i.e., the oil system remains, but operates less frequently), the average annual savings are nearly 3,000 kWh (or about $300).

Do heat pumps produce enough heat for an entire home or business?

Yes. All homes and businesses are capable of being heated and cooled entirely with heat pumps. There are a number of variables a local contractor will take into consideration to recommend the best heat pump option for a home or business. If looking for a whole-home or whole-business heating solution, be sure to let the installer know.

What size heat pump is needed?

Heat pumps will typically be sized larger than standard air conditioners since homes in Michigan have more heating needs than cooling needs. Heat pump sizing varies by home based on square footage of the home, floorplan, insulation levels, air-tightness, etc. Homeowners should talk to their contractor to make sure the heat pump is sized according to their home’s heating and cooling needs. If homeowners want to meet both heating and cooling needs, they should look for a variable speed system.

Is there a temperature below which a conventional heating system is cheaper to run than a heat pump?

As outdoor temperatures get colder and colder, it gets increasingly difficult for a heat pump to absorb heat from the air and the efficiency of the system goes down. On the coldest of days it will typically be cheaper to run a conventional heating system. The exact transition temperature will depend on the heat pump model, electricity and backup fuel prices, and whether the heat pump is sized for heating.

What else can heat pumps do beyond heating?

High-efficiency heat pumps perform the same functions as five different appliances: heaters, air conditioners, dehumidifiers, ceiling fans, and room air filter.

Why do some heat pumps qualify for rebates, and others do not?

High efficiency units have lower operating costs, provide better cooling and dehumidification and may provide more heat at lower temperatures. Homeowners should be sure to tell their contractor how they would like to use these systems to ensure they are sized properly.

## Video Resources

Below is a short list of informative videos that are valuable to better understanding heat pumps and their uses:

What is A Heat Pump? | Extraordinary Electrics | Rewiring America

Heat Pumps (Upper Peninsula Power Company):

The Basics (Part 1 of 4)

How They Work (Part 2 of 4)

Exploring Their Full Potential (Part 3 of 4)

Do They Actually Work in Cold Climates (Part 4 of 4)